Fixation and storage of samples for Flow Cytometric analysis of phytoplankton and bacteria

Outline:

Seawater samples are fixed in 1% (final) paraformaldehyde for 10 minutes at room temperature in a 2 ml capacity cryogenic vial.  The cryovial is then quick frozen in liquid nitrogen.  Storage of the sample can be in the liquid nitrogen or in -80oC freezer.

Procedure:

- Collect seawater sample (i.e. from bucket or flow-through system) and proceed as follows as soon as feasible.

- Prepare duplicate vials from each seawater sample as follows.

- Pipette 1.8 ml of seawater sample into a 2 mL capacity cryogenic vial.

- Then pipette 0.2 ml of 10% paraformaldehyde (PFA) into the vial.  This yields a 1% PFA final concentration. See next page for preparation of PFA.  Remember to filter before use.

- Vortex mix the vial, and let it stand at room temperature for not less than 10 minutes (and not more than about 20-30 minutes).

- Quick freeze the vial in liquid nitrogen.

- The vials may be left in the liquid nitrogen or transferred to an ultra-low temperature freezer (-80oC) for long-term storage.

Paraformaldehyde Solution 10% (This should already be made up – remember to filter before use)

Caution:

Paraformaldehyde is an irritant.  Wear protective clothing and work in a fume hood.

Materials:

Paraformaldehyde

Deionized water

1 N NaOH

1 N HCl

0.2-m pore size filtration unit for plastic syringe

Method:

- Weigh 25 g paraformaldehyde under a fume hood.

- Add 225 ml boiling distilled water.

- Stir vigorously at 70oC for at least 2 hr under a fume hood, until the paraformaldehyde dissolves and saturates the water.

- Add small amounts of 1 N  NaOH drop-wise until the solution becomes clear.

- Let solution cool to room temperature.

- Adjust the pH to 7.4 with 1 N HCl, then make up volume to 250 ml with deionized water.

- Store in brown bottle in refrigerator.

- Just before use, filter through a 0.2-m pore size syringe filter.  This step is essential to remove precipitates from stored PFA.

Queries:  Contact Dr. Bill Li

Bedford Institute of Oceanography, Nova Scotia, Canada

Email: LiB@mar.dfo-mpo.gc.ca