# PAR SENSOR

A PAR sensor will be mounted somewhere on the ship to continuously record the Photosynthetically Active Radiation (PAR).  Data will be logged on a Licor Li 1400 data logger.

The data logger has a low battery warning.  Every week or so, turn the logger ON to check the battery. If there is no warning, turn OFF again.  AA batteries are supplied if batteries need to be replaced.  See manual for replacement procedure.

At the end of each leg, data should be downloaded from the data logger as follows:

- On laptop open LI1400, Click remote then click connect (Check com port, which should be comport 1).

- Click connect, Click remote, Click receive data, Click all, then Click OK.

- Save as file name e.g. Mirai_Leg 1

- Click Save.  It will show the data transferring   - this is slow to start so be patient.  Check the text file then save to floppy.

To erase data on data logger after downloading files:

- Go back to prog, Click remote, Click data base, Click all,  OK,  OK.  It will show synchronizing then finishes.

- This saves the set up file, but erases all data.

- The data logger stays in logging mode throughout this operation.  Reconnect to sensor after downloading and data will be recorded once again.

Queries:  Contact Brian Irwin,

Email:    Brian.Irwin@ns.sympatico.ca